1.Key Features

1, 5 Core functions:

FM transmitter,hands-free, Bluetooth mp3 play,dual USB charging ports,additional cigarette lighter

2, Dual USB ports for your devices charging, built-in microphone

3, With Built-in wireless Bluetooth FM transmitter,enable you to enjoy high quality music and more safe hands free call from car FM stereo while driving.
4, Voice quality:Crystal clear and nature voice based on noise suppression (CVC) technology, A2DP function brings you higher quality sound of stereo music via Bluetooth

5, Works for most of the Bluetooth cellphone on the market: Apple , Samsung, HTC, LG, Sony, Motorola, Nokia and more

- Technical Specification

Rated working voltage: 12V

Limit working voltage: 9~26V

USB output: 5V/2.1A and 5V/1A

Working temperature: 0~50℃

SNR	:>60dB

Degree of distortion: <0．1%

Frequency response: 20Hz-15Khz

Resolution of right and left channel: 〉60dB

Bluetooth version: Version 2.1

Bluetooth sound: A2DP (Advance Audio Distribution

Bluetooth transmission distance: 10M

Effective distance of Bluetooth microphone: 0.5~2M

FM frequency: 87．5~108Mhz

FM Mode: Stereo digital PLL frequency locking

- Instruction

3.1.1Plug the product into cigarette lighter socket and power on the car FM. Adjust the Bluetooth transmitter frequency same as the Car FM’s, then the music in Bluetooth will turned be played from car audio.

3.1.2Ajustment of transmitter frequency:

Long press CH+ and CH- to frequency modulation status, LED will show the frequency point.

Then press CH+ and CH- to adjust the frequency from the point 87.5 to 108MHZ

3.1.3 Long press  for 3 seconds, Bluetooth will be off/on.

## 3.2 Pairing of Bluetooth hands-free and mobile phone

If your phone is connected to Bluetooth hand-free for the first time, match connection is required. Please refer to the match manual of mobile phone and Bluetooth for the detail operation.

- Enable the Bluetooth function of your phone while the Bluetooth hands-free is on and search the “Car kit” device, which will display on your phone.

- Select “Car kit”device

- Select “Car kit”  in the menu of mobile phone, Select Hands-free device as connection in authorized equipment.

4. After power on, Bluetooth hands-free will automatically connect the device paired with in last time

Notice: some phones can automatically connect with Bluetooth car charger with recharging, but some phones may need to confer authority for each connection. You can set “ automatically connection”. For the detail, please refer to the user’s manual of mobile phone.

3.3 using hands-free calls

(When your Bluetooth hands-fee is idle or playing music)

3.3.1 Making a call

Dial the number by pressing the phone number under connection state and the sound will be given out from car audio.

3.3.2 Answering a call

If there is an incoming call under the connection state, press  key to answer the call. In answering the call, press CH+ and CH-key to switch talking between mobile phone and Bluetooth. Short Press  to hang up.

3.3.3 Rejecting a call

For the call you do not want to answer, you may long press  key to reject.

3.3.4 Callback

Press  key twice under connection state to call back the last call.

3.4 Play music in your phone

This product supports A2DP function, which allows you to play music in your phone. Music will be transmitted to your car audio via FM function of the product so that you can enjoy music with higher quality.

3.4.1 According to the mobile phone operation manual, set Bluetooth stereo output as music play mode.

3.4.2 After setting, the music played in mobile phone will be turned to Car audio by FM transmitter.

- Package

1x Bluetooth FM transmitter car charger

1xEnglish User Manual

Reference Link:

http://www.amazon.com/Bluetooth-Transmitter-Lantoo-Handsfree-Smartphone/dp/B01D9YUAAS

http://www.amazon.com/Bluetooth-Transmitter-TheElves-Additional-Cigarette/dp/B01CSB4V4E